## MC-G4  Transfer Switch Fire Pump Controllers		      MASTER

---------- EQUIPMENT ORDERED ----------

COMPLETE CONTROLLER MODEL NO.

IN TYPE 2 DRIP PROOF ENCLOSURE (OR AS INDICATED BELOW)

TYPE 12 DUST TIGHT ENCLOSURE IS STANDARD ON VARIABLE SPEED

WITHSTAND RATED AT 100,000 RMS SYMMETRICAL AMPS AT  200-480VAC (OR AS INDICATED BELOW)

FOR USE WITH        HP.         VAC       PH.        HZ MOTOR

##### STARTING TYPE

MCAT – ACROSS THE LINE				 MCST – SOLID STATE (SCR)

MCPT – PART WINDING				 MCTT - AUTOTRANSFORMER

MCRT – PRIMARY REACTOR				 MCVAT – VARIABLE SPEED, ACROSS-THE-LINE BYPASS

MCOT – WYE DELTA OPEN TRASISTION		 MCVRT – VARIABLE SPEED, REACTOR BYPASS

MCYT – WYE DELTA CLOSED TRANSITION		 MCVST – VARIABLE SPEED, SOFT-START BYPASS

INCLUDES MODIFICATIONS CHECKED BELOW:

(CONTROLLER MODEL NUMBER INDICATES MODIFICATIONS SUPPLIED)

POC IS STANDARD ON VARIABLE SPEED CONTROLLERS

POC – PROGRAMMABLE OPTION CHASSIS – PROVIDES 8 PROGRAMMABLE INPUTS AND 8 PROGRAMMABLE

RELAY OUTPUTS (5 PROGRAMMABLE RELAY OUTPUTS FOR VARIABLE SPEED).   CONTACTS ARE RATED 6 AMPS AT 30 VDC, 6 AMPS AT 250 VAC.  UL PILOT DUTY RATINGS R300  AND B300.  RELAYS CAN BE PROGRAMMED FOR ANY OF THE FOLLOWING: AC FAILURE, AC VOLTS LOW,  CB  TRIP, FAIL TO START, LOAD SHED, LOCKOUT, LOW DISCHARGE PRESSURE, LOW SUCTION PRESSURE, LOW ZONE START (ON DEMAND), MOTOR OVERLOAD,  OVER PRESSURE, PHASE  REVERSAL, PHASESMART PUMP HOUSE TROUBLE INPUTS, PUMP RUN, SINGLE PHASE RUNNING,  TRANSDUCER FAILURE, TSW NORMAL, TSW NORMAL AVAILABLE, TSW ENERGENCY, TSW EMERGENCY AVAILABLE, EMERGENCY CB OPEN.

FC     – FOAM CONTROLLER				 16A  – 500 PSI, PRESSURE TRANSDUCER, TEST VALVE

LPM – LEADING PHASE MONITOR			                  SOLENOID & WET PARTS

SP1   – SUPERVISORY POWER INPUT (120 VAC)		 19     – SPACE HEATER

SP2   – SUPERVISORY POWER INPUT (240 VAC)		 20     – SPACE HEATER WITH THERMOSTAT

12     – TYPE 12 DUSTTIGHT ENCLOSURE		 20A   – SPACE HEATER WITH HUMIDISTAT

3R    – TYPE ER RAINTIGHT ENCLOSURE		 27     – 200,000 AMP EMERGENCY WITHSTAND RATING

4       – TYPE 4 WATERTIGHT ENCLOSURE		 32     – LOW PUMP ROOM TEMPERATURE SWITCH

4XB – TYPE 4X, 304 SS WATERTIGHT ENCLOSURE	                  (REQUIRES OPTION POC)

4XC – TYPE 4X, 316 SS WATERTIGHT ENCLOSURE 	 33      – AUXILIARY POWER OUTPUT (1 AMP @ 120 VAC)

8E    –  CE DECLARATION FOR EUROPEAN COMM.	 83LT – LOW SUCTION TRANSDUCER & WET PARTS

15    – 300 PSI, 316 SS  PRESSURE TRANSDUCER, 		 200    – 200,000 AMP WITHSTAND RATING

TEST VALVE SOLENOID & WET PARTS		 ____________________________________________________

15A – 300 PSI, 300 SERIES SS PRESSURE TRANSDUCER,	 ____________________________________________________

TEST VALVE SOLENOID & WET PARTS		 ____________________________________________________

ORDER ALSO INCLUDES:

JOB IDENTIFICATION:

| aster Control Systems, Inc. LAKE BLUFF, ILLINOIS U.S.A | MC*T ORDER SELECTION DRAWING | Date: 27 February 2014 |
| --- | --- | --- |
|  |  | Drawing 20099 Issue 1 |